| Initial Router Setup for a Cisco 2500 Router The Lab-C router is used here only as an example. Configure each router according to it’s interfaces. These steps assume the router has no startup configuration. | Step 1 |  |
| --- | --- | --- |
|  |  | Lab-C |
|  |  | Router boots up. At the Setup mode prompt, press crtl-c to break out. At the Router> prompt, enter enable for Privileged mode |

| Step 2 Global Configuration Mode Enter global configuration mode Configure hostname Configure enable passwords Configure line passwords Configure interfaces | Step 3 Router Configuration Mode and verify Enter router configuration mode Configure each interface with RIP Exit out of global configuration mode Check your configuration Save your configuration to NVRAM Test your interfaces |
| --- | --- |
| Router# config t Router (config)# hostname Lab-C Lab-C (config)# enable password cisco Lab-C (config)# enable secret class Lab-C (config)# line con 0 Lab-C (config-line)# login Lab-C (config-line)# password cisco Lab-C (config-line)# line vty 0 4 Lab-C (config-line)# login Lab-C (config-line)# password cisco Lab-C (config-line)# line aux 0 Lab-C (config-line)# password cisco Lab-C (config-line)# exit (to leave line configuration) Lab-C (config)# interface e0 (optional for this router) Lab-C (config-if)# ip address 223.8.151.1 255.255.255.0 Lab-C (config-if)# no shutdown Lab-C (config-if)# interface s0 Lab-C (config-if)# ip address 204.204.7.1 255.255.255.0 Lab-C (config-if)# clock rate 56000 Lab-C (config-if)# no shutdown Lab-C (config-if)# interface s1 Lab-C (config-if)# ip address 199.6.13.2 255.255.255.0 Lab-C (config-if)# no shutdown Lab-C (config-if)# exit (to leave interface configuration) Lab-C (config)# Note: Only the DCE interface (s0) on the routers gets the clock rate set. And only set the clock rate on a router interface if you have no other device like a DSU/CSU providing clocking. | Lab-C (config)# router rip (enter the routing protocol to use) Lab-C (config-router)# network 223.8.151.0 Lab-C (config-router)# network 204.204.7.0 Lab-C (config-router)# network 199.6.13.0 Lab-C (config-router)# crtl-z (or end) (exits all the way out of config) Lab-C# show running-config (check config) Lab-C# copy running-config startup-config (save config) Use Ping, Trace and/or Telnet to test connectivity with other routers. Additional commands to check your router’s configuration: Lab-C# show ip interface brief Lab-C# show interfaces Lab-C# show ip route Lab-C# show protocols Lab-C# show startup-config |

b/johnson:01