ERD Commander 2000 - Creating a Bootable CD

Requirements:

- Win2000 or WinNT CD (Workstation or Server)

- ERD Commander 2000 installed on a PC or Server

- CD Burner

- CD Burner Software (this example will use CDRWIN v3.8d)

- Blank CD-R or CD-RW

- If your Server or PC requires disk drivers not included

on the Win2000 or WinNT CD, then you will need to

have them available when running ERD Commander 2000

so they can be included on the bootable CD.

Procedures:

1.  Insert your Win2000/NT CD in your CDROM Drive.

2.  Start ERD Commander 2000 and choose

“Create  a bootable CD Rom”.

3.  Follow the rest of the screens in ERD Commander 2000

to create the directory where the ERD Commander

files will be placed.

4.  Once the directory is created, place a blank

CD in your CD burner and start up your

CD burner software.

5.  For Golden Hawk Technologies CDRWIN

choose, File Backup and Tools:

Note:  To create the bootable ERD Commander 2000 CD, specify the following

settings in your CD Burner software.  The screenshots below will demonstrate

this using CDRWIN.

Use ISO 9660 filesystem (Not Joliet)

Include Hidden files

Include System files

Disable Version Numbers

Set as bootable CD_ROM

Set boot sector image to BOOTSECT.BIN

Set Media Emulation to Custom

Set Load Sector Count to 4

Set Load Segment to 07C0 (hex)

6.  Set the options like the graphic below.

Obviously, your path names will be different.

The File Backup List will be the directory

you created above with ERD Commander 2000.

Note: If your CD Burner supports Burn-Proof

Technology, check that option.  You can

also check the Test option to test the

burn process to make sure it will work.

If the test is successful, uncheck Test before

clicking Start to create the CD.

Once you set these options, click the Advanced

Options button to set your bootable CD options.

7.   The Advanced Options should look like this:

The Bootable Disc tab is the only one you

need to worry about.

Note:   The Image File Name path should point to

the Bootsect.bin file in the directory created with

ERD Commander 2000.

8.  Click OK and then click Start.   You will only be copying

about 5MB of files so the burn process should be quick.

9.  Once finished, test your CD.  Remember, the CD will only

work in PCs / Servers that support booting from the

CDROM drive.

b/johnson:01