Ethernet II Frame:

| 7 bytes | 1 byte | 6 bytes | 6 bytes | 2 bytes | 64-1500 bytes | 4 bytes |
| --- | --- | --- | --- | --- | --- | --- |
| Preamble | Start of Frame Delimiter | Destination Address | Source Address | # Type | # Data | FCS Frame Check Sequence |

802.3 Frame:

| 7 bytes | 1 byte | 6 bytes | 6 bytes | 2 bytes | 46-1500 bytes | 4 bytes |
| --- | --- | --- | --- | --- | --- | --- |
| Preamble | Start of Frame Delimiter | Destination Address | Source Address | # Length | 802.3 Header and Data | FCS Frame Check Sequence |

Preamble – Synchronization.  They give components in the network time to detect the presence of a signal and read the signal before the frame data arrives.

Start of Frame (SOF) – Start of Frame sequence

Destination and Source Addresses – Physical or MAC addresses.  The source address is always a unicast address, the destination address can be unicast, multicast, broadcast.

Length – Indicates the number of bytes of data that follow this field.

Type – Specifies the upper layer protocol to receive the data.

Data – User or application data.  Ethernet II expects a minimum of 46 bytes of data.

If the 802.3 frame does not have a minimum of 64 bytes, padded bytes are added to make 64.

Frame Sequence Check (FCS) – CRC value is used to check for damaged frames.  This value is recalculated at the destination network adapter. If the value is different from what is transmitted, the receiving network adapter assumes that an error has occurred during transmission and discards the frame.

Ethernet Cabling:

| Type | 10Base5 | 10BaseT | 100BaseFl | 100BaseTX | 100BaseFX | 1000BaseT |
| --- | --- | --- | --- | --- | --- | --- |
| ## Media | Thick Coax RG-8, RG-11 | UTP Cat 3, 4, 5+ 4 Pair cable, Uses 2 pair | 62.2/125 micron multi-mode fiber | UTP Cat 5+ 4 Pair cable, Uses 2 pair | 62.2/125 micron multi-mode or single mode fiber | UTP Cat 5+ 4 Pair cable, Uses 2 pair |
| Max Segment Length | 500 Meters | 100 Meters | 2000 Meters | 100 Meters | 400/2000 Meters (full/half duplex) 10,000 Meters (single mode) | 100 Meters |
| Physical Topology | Bus | Star, Extended Star | Star | Star | Star | Star |
| Logical Topology | Bus | Bus | Bus | Bus | Bus | Bus |
| Transfer Rate | 10Mbps | 10Mbps | 10Mbps | 100Mbps | 100Mbps | 1000Mbps |

EIA/TIA Horizontal Cabling:

(Using CAT5 UTP Cabling in an Ethernet network)

## 3 Meters – 90 Meters – 6 Meters

| 3 Meters: Work area patch cable From computer to wall | 90 Meters: What is considered the Horizontal Cabling. From wall outlet to the patch panel in the MDF or IDF (Server Room). | 6 Meters: The patch cable that cross connects a patch panel |
| --- | --- | --- |

.

Collision Domains - A collision domain is defined as a network segment that shares bandwidth with all other devices on the same network segment. When two hosts on the same network segment transmit at the same time, the resulting digital signals will fragment or collide, hence the term collision domain. It's important to know that a collision domain is found only in an Ethernet half-duplex network

Broadcast Domain - A broadcast domain is defined as all devices on a network segment that hear broadcasts sent on that segment.

All devices plugged into a hub are in the same collision domain and the same broadcast domain.

All devices plugged into a switch are in separate collision domains but the same broadcast domain.  Although, you can buy special hardware to break up broadcast domains in a switch, or use a switch capable of creating VLANs.  VLANs breakup broadcast domains.

Hubs and Repeaters extend collision and broadcast domains.

Switches, Bridges and Routers break up collision domains.

Routers (and Switches using VLANs) break up broadcast domains.

| Device | OSI Layer | Filtering | Breaks up Collision Domains | Extends Collision Domains | Breaks up Broadcast Domains | Extends Broadcast Domains |
| --- | --- | --- | --- | --- | --- | --- |
| Hubs | 1 | No |  | X |  | X |
| Repeaters | 1 | No |  | X |  | X |
| Bridges | 2 | Yes | X |  |  | X |
| Switches | 2 | Yes | X |  |  | X |
| Switches using VLANs | 2 | Yes | X |  | X |  |
| Routers | 3 | Yes | X |  | X |  |

b/johnson:01