IP Addressing Guide

Class A first octet range	1 – 127* 	(Network 127 is reserved for loopback and internal testing)		Leading bit pattern	0

Class B first octet range	128 – 191										Leading bit pattern	10

Class C first octet range	192 – 223										Leading bit pattern	110

Class D first octet range	224 – 239	(Reserved for multicast)							Leading bit pattern	1110

Class E first octet range	240 – 255	(Reserved for experimental, used for research)				Leading bit pattern	11110

# Maximum number of unique addresses in each class

Class A	  27 – 2 = 126		           Class B	214 – 2 = 16,382			Class C	  221 – 2 = 2,097,150

# Formulas to determine the number of subnets or hosts created by borrowing from the host portion of the address

M = the number of bits borrowed (Masked)		U = the number of bits remaining or not borrowed (Unmasked)

2M = Total # of subnets created				2U = Total # of host on each subnets

2M – 2 = Useable # of subnet created			2U – 2 = Useable # of host on each subnet

| Class A License |  |  | ## N.H.H.H |  |  |  |  | Class B License |  |  | N.N.H.H |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of Bits Borrowed | Subnet Mask | Total # Subnets | Useable # Subnets | Total # Hosts | Useable # Hosts |  |  | # of Bits Borrowed | Subnet Mask | Total # Subnets | Useable # Subnets | Total # Hosts | Useable # Hosts |
| 2 | 255.192.0.0 | 4 | 2 | 4,194,304 | 4,194,302 |  | 2 | 255.255.192.0 | 4 | 2 | 16,384 | 16,382 |
| 3 | 255.224.0.0 | 8 | 6 | 2,097,152 | 2,097,150 |  | 3 | 255.255.224.0 | 8 | 6 | 8,192 | 8,190 |
| 4 | 255.240.0.0 | 16 | 14 | 1,048,576 | 1,048,574 |  | 4 | 255.255.240.0 | 16 | 14 | 4,096 | 4,094 |
| 5 | 255.248.0.0 | 32 | 30 | 524,288 | 524,286 |  | 5 | 255.255.248.0 | 32 | 30 | 2,048 | 2,046 |
| 6 | 255.252.0.0 | 64 | 62 | 262,144 | 262,142 |  | 6 | 255.255.252.0 | 64 | 62 | 1,024 | 1,022 |
| 7 | 255.254.0.0 | 128 | 126 | 131,072 | 131,070 |  | 7 | 255.255.254.0 | 128 | 126 | 512 | 510 |
| 8 | 255.255.0.0 | 256 | 254 | 65,536 | 65,534 |  | 8 | 255.255.255.0 | 256 | 254 | 256 | 254 |
| 9 | 255.255.128.0 | 512 | 510 | 32,768 | 32,766 |  | 9 | 255.255.255.128 | 512 | 510 | 128 | 126 |
| 10 | 255.255.192.0 | 1,024 | 1,022 | 16,384 | 16,382 |  | 10 | 255.255.255.192 | 1,024 | 1,022 | 64 | 62 |
| 11 | 255.255.224.0 | 2,048 | 2,046 | 8,192 | 8,190 |  | 11 | 255.255.255.224 | 2,048 | 2,046 | 32 | 30 |
| 12 | 255.255.240.0 | 4,096 | 4,094 | 4,096 | 4,094 |  | 12 | 255.255.255.240 | 4,096 | 4,094 | 16 | 14 |
| 13 | 255.255.248.0 | 8,192 | 8,190 | 2,048 | 2,046 |  | 13 | 255.255.255.248 | 8,192 | 8,190 | 8 | 6 |
| 14 | 255.255.252.0 | 16,384 | 16,382 | 1,024 | 1,022 |  | 14 | 255.255.255.252 | 16,384 | 16,382 | 4 | 2 |
| 15 | 255.255.254.0 | 32,768 | 32,766 | 512 | 510 |  |  |  |  |  |  |  |
| 16 | 255.255.255.0 | 65,536 | 65,534 | 256 | 254 |  | Class C License |  |  | N.N.N.H |  |  |  |
| 17 | 255.255.255.128 | 131,072 | 131,070 | 128 | 126 |  | # of Bits |  | Total # | Useable # | Total # | Useable |
| 18 | 255.255.255.192 | 262,144 | 262,142 | 64 | 62 |  | Borrowed | Subnet Mask | Subnets | Subnets | Hosts | # Hosts |
| 19 | 255.255.255.224 | 524,288 | 524,286 | 32 | 30 |  | 2 | 255.255.255.192 | 4 | 2 | 64 | 62 |
| 20 | 255.255.255.240 | 1,048,576 | 1,048,574 | 16 | 14 |  | 3 | 255.255.255.224 | 8 | 6 | 32 | 30 |
| 21 | 255.255.255.248 | 2,097,152 | 2,097,150 | 8 | 6 |  | 4 | 255.255.255.240 | 16 | 14 | 16 | 14 |
| 22 | 255.255.255.252 | 4,194,304 | 4,194,302 | 4 | 2 |  | 5 | 255.255.255.248 | 32 | 30 | 8 | 6 |
|  |  |  |  |  |  |  | 6 | 255.255.255.252 | 64 | 62 | 4 | 2 |

NOTE:  You must borrow at least 2 bits and you must leave at least 2 bits.

G. Wright 	Education Service Center – Region XI,   Ft. Worth, TX		glwright@esc11.net